ITCS 3166

Assignment 2

Note: Deadline for this assignment is Sep. 27 by 11:59 PM

- The following character encoding is used in a data link protocol:

A: 01000111 	B: 11100011 		FLAG: 01111110 		ESC:11100000

Show the bit sequence transmitted (in binary) for the four-character frame A B ESC FLAG when each of the following framing methods is used:

- Byte count.

00000100 01000111 11100011 11100000 01111110

- Flag bytes with byte stuffing.

01111110 01000111 11100011 11100000 11100000 11100000 01111110 01111110

- Starting and ending flag bytes with bit stuffing.

01111110 01000111 110100011 11100000 011111010 0111110

- A bit string, 0111101111101111110, needs to be transmitted at the data link layer. What is the string actually transmitted after bit stuffing?

011110111110011111010

- One of your classmates, Scrooge, has pointed out that it is wasteful to end each frame with a flag byte and then begin the next one with a second flag byte. One flag byte could do the job as well, and a byte saved is a byte earned. Do you agree?

No, cause if one flag byte is enough then the count of the frame is endless stream, which would cause more unnecessary time to start a new frame and also flag bytes are added at end of current frame and start of a new frame to keep in sync with receiver.

- What is the maximum overhead in byte-stuffing algorithm?

The maximum overhead in byte-stuffing algorithm is

2n

n = the num of bytes in payload

- A channel has a bit rate of 4 kbps and a propagation delay of 20 msec. For what range of frame sizes does stop-and-wait give an efficiency of at least 50%?

                - 160 bits